M.16.08

Article M.16.08 - Pedestrian Push Button:

For RRFB (Rectangular Rapid Flashing Beacon):

Delete the entire section and replace with the following:

A. General

- Size and force compliant with ADA, Section 14.2.5, Crossing Controls.

- Tamper-proof, and Vandal-proof, Weatherproof, Freeze-proof, Impact-resistant design and construction.

- Completely insulated to preclude electrical shock under any weather conditions.

- Wire entrance through the rear.

- Stainless steel mounting hardware.

- Any features in conflict with this spec and MUTCD shall be disabled.

- Accessible features shall:

- conform to the MUTCD

- provide the speech message “yellow lights are flashing” that shall be spoken twice

- include tactile arrow on the button

- include a locator tone

- not use vibrotactile indications or percussive indications for the audible information

  - The vibrotactile feedback shall be disabled.

- Bluetooth® Low Energy level 5 radio to provide wireless communication features.

- Operation programming method: Either or combination of:

- Windows Laptop Computer with USB Cable

- iOS Device and Bluetooth Wireless Connection

- Android Device and Bluetooth Wireless Connection

B. Audible Pushbutton Locator Tone

- Frequency: repeating tone at one (1) second intervals

- Tone duration: ≤  0.15 seconds

- Volume:

- Minimum setting of zero

- Manually adjustable initial setting

- Automatically adjusted after initial setting. Volume increased in response to a temporary increase in ambient noise and subsequently decreased with a decrease in ambient noise.

- Maximum volume: 100 dBA which is the approximate sound pressure of a gasoline powered lawn mower nearby.

- Automatic volume adjustment independent of other Audible Information Devices at the intersection.

- Should not be disabled but have the capability of being disabled without affecting operation of other features.

- Silent only when RRFB yellow lights are flashing.  Active all other times.

C. Tactile Arrow Pushbutton:

- Pushbutton contained in a circular assembly which fits inside the housing and is attached to the housing with 4 screws.

- The confirmation light shall flash simultaneously with the RRFB indications.

- ADA compliant: Size: ≥  2.0” diameter, Actuation force: ≤ 5 ft-lb

- Shape: Circular, raised slightly above housing so that it may be actuated with the back of a hand

- Tamper-proof, vandal-proof, weatherproof, freeze-proof, impact-resistant design and construction.

- Tactile Arrow:

- Attached to surface of the button assembly by a tamperproof method.

- Raised slightly above surface of pushbutton, minimum 0.125”

- Size:  Length ≥  1.5”, Height ≥  1.0”

- Color: Sharp contrast to background color of pushbutton and housing

D. Housing

- Frame and button cover made of die cast aluminum meeting requirements of ASTM B85.

- Housing made of reinforced, UL-listed thermoplastic or aluminum. Designed to attach 9" x 12" four-hole advisory sign.

- Flat back to facilitate surface mount.

- Mounting plate made of aluminum mounts to pole with two ¼”-20 stainless steel bolts.  Mounting plate separates from the button during installation and maintenance.

- Available extension bracket of a size indicated on the plan – 18” maximum.

E. Painting

- Finish: Cover/Frame and all mounting brackets either:

1.  Painted with 3 coats of infrared oven-baked paint before assembly.

- Primer: Shall be iron oxide baking primer and shall meet or exceed performance

- specification of FS TT-P-664.

- Second coat: Exterior-baking enamel, light gray, which meets or exceeds FS TT-E-527.

- Third coat: YELLOW exterior-baking enamel and shall meet the requirements of FS A-A 2962. The color shall be AMS-STD-595 Color No. 13538.

- Electrostatic powder coated after chemically cleaned. Pre coating material prep in accordance with ASTM_D1730-09 to include all parts sent through a multi-stage phosphate wash process and preheat prior to any application of powder coat.