TECH NOTES (revised 9/29/2022)

PEDESTRIAN RELATED

TECH-1P	COUNTDOWN ONLY DURING FLASHING PEDESTRIAN CHANGE INTERVAL.

TECH-2P	PERCUSSIVE TONE ONLY DURING PEDESTRIAN WALK INTERVAL.

TECH-3P	CONCURRENT PED PHASE ONLY COMES IN WHEN ACTUATED BY ASSOCIATED PUSH BUTTON

TECH-4P	SPEECH MESSAGE “(INSERT STREET NAME).  WALK SIGN IS ON TO CROSS (INSERT STREET NAME)”. DURING PEDESTRIAN WALK INTERVAL.  (used where speech message is needed on both sides of the associated crosswalk)

TECH-5P	PERCUSSIVE TONE ONLY DURING PEDESTRIAN WALK INTERVAL FOR P# BUTTON(S).  SPEECH MESSAGE “(INSERT STREET NAME).  WALK SIGN IS ON TO CROSS (INSERT STREET NAME) DURING PEDESTRIAN WALK INTERVAL FOR P#A BUTTON(S).  (used where percussive tone is possible on one side of crosswalk and speech message is needed on the other side of the crosswalk)

COORDINATION/SYSTEM RELATED

TECH-1S	ARTERY DETECTORS TO BE NON-ACTUATING DURING COORDINATION.

TECH-2S	TIMINGS SHOWN REFLECT FREE OPERATION.

TECH-3S	ACTUAL COORDINATION INFORMATION TO BE DETERMINED BY THE COORDINATION SYSTEM.

TECH-4S	SIGNAL MAY DOUBLE CYCLE IF PEDESTRIAN PHASE IS CALLED.

TECH-5S	UNUSED TIME TO BE REALLOCATED.