ITEM #0979005A – PEDESTRIAN BARRICADE

Section 9.79 is supplemented and amended as follows:

9.79.01—Description:

Replace the entire Article with the following:

Under this item the Contractor shall furnish all Pedestrian Barricades required on the Project as stated in the item “Maintenance and Protection of Traffic,” as shown on the Plans, and as directed by the Engineer.

Pedestrian barricades visible to vehicular traffic shall have retroreflective sheeting compliant with the Retroreflective Sheeting: Construction Barricade Sheeting section of the Department’s Qualified Product List on the side(s) facing vehicular traffic.

9.79.02—Materials:

Replace the entire Article with the following:

Pedestrian Barricades used on the Project shall be listed on the Department’s Qualified Product List under the Pedestrian Barricade category as follows:

- The barricades listed under the Test Level 3 category may be used on roadways with a posted speed limit of 60 miles per hour or less;

- The barricades listed under the Test Level 2 category may be used on roadways with a posted speed limit of 40 miles per hour or less;

Water filled Pedestrian Barricades shall only be used when the air temperature is above 33ºF.  Such barricades shall only be filled with water and may not contain additional chemicals.

9.79.03 - Construction Methods:

Add the following:

Pedestrian Barricades shall be installed per manufacturer guidelines.

Barricade supports shall not protrude more than 4 inches into the pedestrian facility.

Pedestrian Barricades used to close a pedestrian facility shall cover the entire width of the pedestrian facility.

Pedestrian Barricades used for channelizing pedestrian traffic shall meet the following requirements:

- Barricades along the pedestrian path shall be interlocked and the gap between adjacent devices shall not exceed 1 inch;

- Barricades shall be installed to produce a smooth continuous length of barrier, except for locations where access to, or crossing, a pedestrian facility is allowed;

9.79.04—Method of Measurement:

Replace the entire Article with the following:

Pedestrian Barricades will be measured as the number of feet along the top rail of pedestrian barricades installed and accepted on the Project.

9.79.05—Basis of Payment:

Replace the entire Article with the following:

Pedestrian Barricades will be paid at the Contract unit price per linear foot for "Pedestrian Barricade."  The length of Pedestrian Barricades will be paid for once, regardless of the number of times used on the Project.

Pay Item			Pay Unit

Pedestrian Barricade		l.f.