ITEM NO. 0980020A - CONSTRUCTION SURVEYING

Section 9.80 Construction Surveying is supplemented as follows:

Article 9.80.03 – Construction Methods

In Subarticle IV Specific Requirements, add the following:

13.   Traffic Signals: The Contractor shall survey and stake proposed traffic signal mast arm, span pole, and cabinet foundations as called on the Contract Plans.  Proposed foundation locations are given with GPS coordinates on the plans.  The Contractor shall request the Engineer to contact CT DOT District Surveys Supervising Engineer of the relevant district, two weeks prior to surveying and staking the foundations in order to schedule verification of staked equipment locations.  The Contractor shall maintain or replace these stakes until the Engineer approves their removal.  No excavation of mast arm, span pole, or cabinet foundations shall take place until the foundation locations have been verified by District Surveys and approved by the Engineer.

As required by Section 1.08.03 of the Contract and before any walk through(s), the Contractor shall also have the construction area surveyed and marked out by a professional underground utility locator. Any locator marking the location of underground facilities on behalf of a public utility shall be trained in applicable locating industry standards and practices equal or superior to the National Utility Locating Contractors Association’s locator training standards and practices. Each locator’s training shall be documented, and such documents shall be provided to the public utility.

For Projects that only consist of removal/replacement of pavement markings and/or traffic signals, the requirement for a licensed surveyor to supervise the staking is waived.