ITEM #1113901A — CAMERA CABLE

Description:  Furnish and install Camera Cable continuous between the Video Detector Assembly (VDA) and Video Detection Processor (VDP).

Materials:

Camera Cable:

- Supply the VDA power and return the video signal to the VDP.

- Composite construction or as recommended by camera manufacturer.

- Coaxial:

- 20 AWG, solid conductor.

- Polyethylene foam dielectric.

- Minimum 95% bare copper braid shield.

- Power/ Control:

- 5 conductors 18 AWG, 7 strand conductor, shielded.

- Polyethylene or polyvinyl chloride jacket.

- Other type cable may be substituted at the request of the VDP manufacturer.

- Connectors:

- Use compression type connectors with compression tool to make connections.

Method of Measurement:

Camera Cable will be measured for payment by the number of linear feet of cable furnished, installed and accepted.

Basis of Payment:

The Contract price per linear foot of “Camera Cable” shall include all connectors, labor, tools and equipment necessary to install the cable between the VDA and the VDP.

## Pay Item						Pay Unit

Camera Cable						l.f.