ITEM #1116110A - BLANK-OUT SIGN

Section 11.16 is amended as follows,

# 11.16.01 - Description:

This item shall consist of furnishing a MUTCD Compliant Internally Illuminated

Blank-Out Sign. Blank-Out Sign shall be a single or dual message sign as specified on associated Traffic Control Signal Plan.

# 11.16.02 - Materials:

The materials for this work shall conform to the following specifications.

The Blank-Out Sign will have independently powered and controlled messages. The power

modules are compatible with all Traffic LED display panels and can be replaced while powered.

The front panel assembly, as well as the main components, may be replaced using just a

flat screwdriver, which facilitates upgrading and maintenance operations.

The Blank-Out Signs shall be fully compatible with:

- Standard traffic signal conflict monitors (NEMA and 270)

The signs shall meet or exceed the following specifications and comply with current MUTCD

standards:

- Legend: Shall be as specified on the Signal Plan

- Text and Symbol Size: Shall substantially conform to the corresponding FHWA Standard Highway Signs detail. No Turn On Red shall use white LEDs.

- Painted aluminum Visor

- 0.188 Non-glare Polycarbonate Lens with UV resistant surface treatment.

- Top and Bottom serrated Hub ready to be installed with mounting support.

- Waterproof aluminum enclosure that meets NEMA requirements for type 4 enclosures

- Compliance with Institute of Transportation Engineers (ITE) requirements applying to

LED road signs

- Exterior dimensions:

  - Size shall be as per Signal Plan

  - Depth: 2″-4”

- Supply voltage: 90–135 VAC/60 Hz

- Rear Mount NEMA4x Power Supply

- Maximum power: 30 W; nominal power: 15 W

- Power factor > 90%

- Compliance with operating temperature criteria of NEMA TS2 standard 30 to +165°F

- LEDs of stable brightness and chromaticity over the entire power and temperature ranges

- All housing, brackets and attachment hardware shall be black.

11.16.03 - Construction Methods:

Install the Blank-Out Sign in accordance with the manufacturer instructions and

recommendations, and in accordance with MUTCD standards.

11.16.04 - Method of Measurement:

This work will be measured for payment by the number of Blank-Out Signs, either single or dual message, completed, installed, and accepted in place.

# 11.16.05 - Basis of Payment:

This work will be paid for at the contract unit price each for “Blank-Out Sign” of the type specified, either single or dual message, which price shall include all materials, brackets, miscellaneous hardware, labor, tools, and work incidental thereto.

Pay Item                                Pay Unit

Blank-Out Sign	      Ea.